How to release a TIFF drawing to document control
By Scott Wallters

7-7-05

For Protel:

1. Load the Tiff Driver. It’s located on SLAC website under computing/software/xweb/SLAC tiff release driver.
2. Go to Start/Programs/Slac tiff release driver/VB printer settings.

3. In box “filename generation method” select “exact filename”  Type in name of drawing using no spaces, with a “.tif” extension at the end. Example: (sd44480563c0s1.tif ).
4. Make sure your drawing size is selected in the paper size box.

5. In Graphic resolution box, make sure “medium resolution 300 x 300DPI” is selected. Hit Ok to close.
6. Bring up your Protel drawing that is ready to sign off.

7. Go to file/setup printers/properties and select paper size of your drawing. A D-size drawing template requires that D-size paper be selected (no oversize required).
8. Under properties/printers, select the “SLAC Tiff Release”.

9. Under printer set up main menu, in the scale box should show a percentage slightly smaller that 100% ( could be around 99.941% for a E-size). For some reason Protel will try to print out 4 sheets of a E-size if 100% is selected, but if it’s less that 100% it prints out 1 sheet only. Margins should be at zero. Hit OK to close.
10. Go to file/print and print the drawing to “SLAC release driver”.

11. In a short time a window will appear with the document and a long Tiff release pathname placed in the lower right hand corner of the drawing template just below the SPIRES title. If the path name is NOT in this position inside the template, but below it, the drawing will fail! To correct this you need to make sure all the printer settings are selected for proper paper size. Hit OK to close
12. To print out a hard copy to sign perform the following: Go to Start/programs/Slac tiff release driver/MD plot images. Select your drawing number from list, and hit print.

13. From print menu, select you favorite plotter and under preferences select a paper size you want. Document control does not care if it’s a E-size or a B-size, just as long as they can read the Tiff release path name at the bottom of drawing. This paper copy will have a time stamp that “sync’s” up with electronic copy. Document control needs to match the time stamp of the paper vs. electronic copy to pass the drawing. All you need to known is don’t re-plot a tiff image after a paper plot. 
14. Have the paper copy of drawing signed, fill out the document control form in copy room of B137 and drop off on Stacey’s desk to release.

15. Drop your electronic protel file in the pending folder. The drawing number can have dashes, but needs to be all lower case with a  _protel after the extension. The extension is masked in Microsoft XP so you won’t be able to see it.

For Autocad:

1. Go to Start/programs/Slac tiff release driver/VB printer settings. In box “Filename generation method” choose “use the document name” option.

2. Follow above procedure.

